
(Suppli

	r Audit)

	
		審核

	檢查

	制作

			罗 强
	梁虹杏

		日期：   2014 年  05 月 20日

版次：A


	[image: image1.png]



供应商名称：深圳福田区锦威电子
经销部

供应商厂址：江苏省常州市新闸镇庆路57号工业园

供应商展址：深圳市华强北华强电子世界三号楼一楼A234

电话：0755-83687275    13828805336    

传真：0755-83687275/82981792

E-mail:CYH336@126.COM




	Model  No.
	4020G8-01
	Date
	2014-5-20
	Page No.
	1 / 6

	產品規格內容 (SPECIFICATION)

1.使用範圍 (Scope)

此份規格書是針對多媒體產品內，所使用的微型揚聲器單體所提出的報告.

This specification covers our product of miniature speaker unit for multimedia product.
2.标准测试条件(Test condition)
        一般规格试验状态（特殊要求除外）：常温5~35℃，常湿45%~85%，气压860~1060hPa.

          判定上有疑问时，试验状态为：温度20±2℃，相对湿度60%~70%，气压860~1060 hPa.

          The test conditions unless otherwise specified should be normal temperature 5 to 35 ℃,Normal humidity 45 to 85% and normal atmospheric pressure 860 to 1060 hPa,However,if there arises a      doubt in judgment,the test conditions shall beas follows:

Temperature   20±2℃

Hunidity       60%~70%
Atmosphere   860~1060 hPa
3.電聲特性 (ELECTRO-ACOUSTIC CHARACTERISTICS)
   3.1額定阻抗(Rated Impedance):

       8Ω ± 20%   (AT 1.6K Hz ,1.0 Vrms)
   3.2額定輸入功率(Rated Input Power)
       1.0 Watt    (At  2.83 Vrms white noise 96 Hours)

   3.3最大輸入功率(Max. Input Power)
       2.0 Watt
   3.4最低共振頻率(Resonant Frequency)
       800 Hz ± 30%
   3.5輸出音壓(Output S.P.L)
      93 ± 3dB   (1.0 Watt , 0.1 Meter. Average at 1.0K,1.2K,1.5K,2.0K Hz)
   3.6實效頻率範圍(Frequency Range)
      Fo ~ 10KHz  (By Output S.P.L. –10dB)

   3.7失真率(Distortion)
      Less than 5%  (At 1.6KHz ,2.0 Vrms)
   3.8工作溫度(Operating Temperature)
       -25℃ ~ +50℃
  3.9儲存溫度(Storage temperature)
       -25℃ ~ +55℃

	制    定
	审    核
	核    准
	文  件  编 号

	Xuan xing xian
	罗裕强
	
	


產品規格內容 (SPECIFICATION)
	Model  No.
	4020G8-01
	Date
	2014-5-20
	Page No.
	2 / 6

	4.可靠性測試 (Reliability Test)
   4.1高低溫差循環測試(Temperature Cycling Test)
[image: image9.jpg]1 Cycle




      
A

Low Temperature (-25℃±3℃)

B

High Temperature (+55℃±3℃)

C

Number of Cycle (3 Cycles)

D

Duration of Exposure 1 Hours, at Each Temperature Extremes

E

Duration of Recovery 1 Hours 

    4.2抗濕度試驗(Humidity Test)
A

相對濕度(Relative Humidity)

90-95%  at  + 40℃ ± 3℃
B

測試時間(Durations)

96 Hours

C

恢覆時間(Durations of recovery)

6 Hours 

    4.3連續負載測試(Load Test)
        White Noise Without Weighted Filter 2.83 Volts.(RMS) 100 Hrs.
    4.4跌落試驗(Drop Test)
       75 cm Free Falling On Concrete Floor, 10 Times.
    4.5異常音測試(Buzzes & Rattles)
       Sine wave between at 2.83 Vrms / 50~5000Hz
    4.6極性測試(Polarity test)
      Diaphragm Shall Move Forward When Apply a Positive DC.  Current to the “+”or “Marked” Terminal.



產品規格內容 (SPECIFICATION)
	Model  No.
	4020G8-01
	Date
	2014-5-20
	Page No.
	3 / 6

	5.測量方式 (Measurement Method)

[image: image2.png]SPEAKER

socm

tm mnﬂ

s

e

Powtr FmaliFTer

Audio Analyzer

Lo





[image: image3.png]67.7cm

Vg
. ['e]
Speaker| &
| 2
o
= g
3
- 9
o
el








產品規格內容 (SPECIFICATION)
	Model  No.
	4020G8-01
	Date
	2014-5-20
	Page No.
	4/6

	6. 頻率響應曲線圖(Frequency Response Curve)
[image: image4.jpg]FYIRERIRT AR AR Sinusoidal 2010-4-12 1506 49

110.0 o T1800

dBSPL Deg
100.0 108.0
90.0 36.0
80.0 -36.0
70.0 -108.0
60.0 -1800

10 100 1k 10k Hz 20k

CHA dBSPL Resolution 1/6 Octave 1/3 Octave Delay[ms] 0.000 Dist Rise [dB] 30.00




7. 阻抗曲線圖(Impedance Curve)
[image: image5.jpg]FRAIAT AR AR Sinusoidal 2010-4-12 145854
15.00 o T1800
ohm Deg
13.00 108.0
1100 36.0
9.00 -36.0
| A1
7.00 -108.0
500 -1800
10 100 1k 10k Hz 20k

CHA Ohm Resolution 1/6 Octave 1/3 Octave Delay[ms] 0.000 Dist Rise [dB] 30.00







產品規格內容 (SPECIFICATION)
	Model  No.
	2040-8R
	Date
	2014-5-20
	Page No.
	5 / 6

	8. 喇叭外觀描述
（Unit:mm  tolerance:±0.3）

[image: image6.png]o
A
N

N





[image: image7.png]80+3
80+3

L

Y(: L= 80 un WIRE:UL1571 £30 1.25-2P






產品規格內容 (SPECIFICATION)
	Model  No.
	4020G8-01
	Date
	2014-5-20
	Page No.
	6/6

	10.包裝示意圖(Packing Standard):  
（Unit:cm tolerance:±3）

[image: image8.png]





_1301148772

_1306762999

