
SPECIFICATION  FOR  APPROVAL
承    认    书
Description      

     :       
 Integrated Buzzer 
Vender’s Part  No.      :        TMB12A12     
Customer’s　Name.
   : 
Serial  No. 


      :         

Version  No.           
:            

	CUSTOMER’S APPROVED SIGNATURE

	
	
	



	Approved By
	Checked By
	Made  By

	
	
	


A. SCOPE 
   This specification applies 
Integrated buzzer,TMB12A12
B. SPECIFICATION 
	No.
	Item
	Unit
	Specification
	Condition

	1
	Oscillation Frequency
	KHz
	2.5±0.2
	

	2
	Operating Voltage
	VDC
	8-15
	

	3
	Rated Voltage
	VDC
	12
	

	4
	Current Consumption 
	mA
	MAX 25
	at Rated Voltage

	5
	Sound Pressure Level
	dB
	MIN. 85
	at 10cm at Rated Voltage

	6
	Tone Nature
	
	Constant
	

	7
	Operating Temperature 
	℃
	-30~ +75
	

	8
	Storage Temperature
	℃
	-40~ +85
	

	9
	Dimension
	mm
	Φ12.0 x H9.5
	See appearance drawing

	10
	Weight (MAX)
	gram
	2.0
	

	11
	Housing Material
	
	PPO( Black )
	

	12
	Leading Pin
	
	Triode
	

	13
	Environmental  
Protection Regulation
	
	 \
	


C. APPEARANCE DRAWING
    Tol: ± 0.5     Unit: mm [image: image1.jpg]



D. Profile Picture:
        [image: image2.jpg]



   E. Typical Frequency Response Curve
         [image: image3.jpg]B R 8 G

dB-V 8-V
o8 T I
o
£
- 1 =
o @ /5:
A Temp 25| W\ Tormp 25 ! Tomp:
ooy LT Lofb)
1214 15 18 20V 34567 810 2 14 eV
mAY mAY mAY
MA‘ ™
30 Ed
= x5! x
I
Témp| ] 2 el 25 ol Tefp 25T
st 5
2l | Pl !
127141618 20v 34567 & 510 12 14 18y




F .可靠性测试
在下面任何一项测试后都要符合以下标准，外观和电性能(除声压外)上不能有不良或衰减现象出现，声压不能低于规格说明书的最低声压的10dB。
1.1  寿命测试
将测试件置于25±10℃的温度下，输入额定电压、占空比为1/2的谐振频率方波测试96小时。
1.2  高温测试
将测试件置于+85℃的温度下存放96小时，取出置于常温下3小时而后测试。
1.3  低温测试
将测试件置于-40℃的温度下存放96小时，取出置于常温下3小时而后测试。
1.4  湿度测试
将测试件置于温度+40±3℃,湿度90%～95%的条件下持续48小时，再置于室温下6小时而后测试。
1.5  温度冲击测试
将测试件置于以下温变条件下：70℃2小时→-25℃2小时(1周期)，做完一个周期实验，过1个小时后再继续下一次实验，总测试周期数: 10个周期。
1.6  落地测试
将测试件标准包装后(包装盒的3个边6个面)从1.2米的高度下跌落到厚木板或5厘米厚的硬木板上。
1.7  机械振动测试
按以下条件实验后再测试：振速:1000次/分；振幅:1.5毫米；时间: 1小时更换一个自由度方向共3个自由度方向。
注意:
需避免无关材料进入产品内，无关材料如：磁性材料、水溶性的材料、腐蚀性的气体等。因为这些无关材料将会影响产品的电性能。
 

G.电性能及声音测试条件
          测试装置
 [image: image4.jpg][
Poger

C)

WzZzeT

40cn

| Loen |

L

/ |

Bz

SPL

Frequency




 

测试电路
 [image: image5.jpg]1590

lis

i

Hin 100ms

0

)

N 16




 



















PAGE  

