
[image: image4]
DATE: 2017-04-08        SPEC. No.: JHY-1510-50           Total：7 sheet

SPECIFICATION 

            SHEET FOR AP  PROVAL

                        (Revision：2017S Update: 00)
CUSTOMER:  
PRODUCTS: MOBILE SPEAKER

MODEL NUMBER: JHY-1510-50
CUSTOMER NUMBER:

 INTERNAL DESCRIPTION:
“JHY-1510-50”  15×10 Racetrack-type   H3.5mm  8Ohm   FRAME: Plastic
ENVIRONMENT REQUEST：

□NON-ROHS             ■ROHS            □OTHERS

	
	
	

	ENGINEER
	CHECKED
	APPROVED


	
	

	SIGNATURE
	IMPRINTMENT


 CUSTOMER  APPROVED           

REVISION   HISTORY

	NO.
	Revision
	Update
	Comment
	Page No.
	Date

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	


	SPECIFICATION
	MODEL NO.
	JHY-1510-50
	P1/5

	ISSUED DATE
	2017-04-08
	
	
	

	REVISION
	2017S
	UPDATE
	00
	

	  1. SCOPE 
 This specification covers our product of dynamic speaker unit for mobile telephone use.
  2. MECHANICAL LAYOUT & DIMENSIONS

     Shown in Fig.8

  3. GENERAL REQUIREMENTS

3.1 OPERATING TEMPERATURE RANGE: -30℃ ~ +70℃
3.2 STORAGE TEMPERATURE:          -40℃ ~ +85℃
3.2 STANDARD TEST CONDITIONS:

    Temperature:                17～25℃ 
    Relative Humidity:           45%～80%(RH)
    Air Pressure:                860～1060 hPa
3.3 JUDGEMENT CONDITIONS:

        Temperature:                 20±2℃

        Relative Humidity:            60%～70%(RH)

   Air Pressure:                 860～1060 hPa

4. SPEAKER MODE   
     4.1 SOUND PRESSURE LEVEL   

        82±3dB SPL @1.5 ,2.0 ,2.5, 3.0kHz in average (0dB SPL=20μPa)

        Measuring condition: 0.1W (Sine wave) 0.1m measured with baffler shown in Fig.4.

4.2 IMPEDANCE: 8 ±15% Ω(@1kHz 1V) without baffler.

4.3 MEASURING DIAGRAM: Shown in Fig.4.

4.4 TYPICAL FREQUENCY RESPONSE CURVE: Shown in Fig.5.

4.5 RATED POWER: 0.8W.  MAX POWER:1.0W

4.6 RESONANCE FREQUENCY (F0):1050±20%Hz @ 1V. (Without Baffler)




	SPECIFICATION
	MODEL NO.
	JHY-1510-50
	P2/5

	ISSUED DATE
	2017-04-08
	
	
	

	REVISION
	2017S
	UPDATE
	00
	

	  ■ FREQUENCY MEASURING CIRCUIT (Fig.4)
[image: image5.wmf]Microtech standard anechoic room

Measuring microphone

B & K 4191 

Test speaker

Measuring distance: 0.1M

Audio Power Amplifier

B & K 2716C

Audio Analyzer

B & K 3560C


Fig.4 Illustration of measuring diagram (speaker mode)
■ TYPICAL FREQUENCY RESPONSE CURVE  (Fig.5) 
[image: image1.wmf]lao ban2 lanson (Magnitude)

Working : Input : Input : SSR Analyzer

63

125

250

500

1k

2k

4k

8k

16k

30

40

50

60

70

80

90

100

110

[Hz]

[dB/20.0u Pa]

lao ban2 lanson (Magnitude)

Working : Input : Input : SSR Analyzer

63

125

250

500

1k

2k

4k

8k

16k

30

40

50

60

70

80

90

100

110

[Hz]

[dB/20.0u Pa]


Fig.5 Typical frequency response curve



	SPECIFICATION
	MODEL NO.
	JHY-1510-50
	P3/5

	ISSUED DATE
	2017-04-08
	
	
	

	REVISION
	2017S
	UPDATE
	00
	

	6. RELIABILITY TESTS 
The sound pressure as specified shall neither deviate more than ±3dB from the initial value, nor any significant damage after any of following testing.

	6.1 HIGH TEMPERATURE TEST

   High temperature:              +65±2℃  
   Duration:                      96 hours

	6.2 LOW TEMPERATURE TEST
   Low temperature :              -25±2℃  
   Duration:                      96 hours

	6.3 HUMIDITY TEST
   Temperature:                  +30±2℃
   Relative humidity:              90～95%
   Duration:                      96 hours


	6.4 TEMPERATURE CYCLE TEST (See in Fig.6)
   Temperature:                  -25℃          +65℃
   Duration:                     45 minutes     45 minutes

   Temperature gradient:           1~3℃/min.
   Cycle:                        6



	6.5 LOAD TEST
   White noise (EIA filter) for 24 hours @0.8W input power.




	SPECIFICATION
	MODEL NO.
	JHY-1510-50
	P4/5

	ISSUED DATE
	2017-04-08
	
	
	

	REVISION
	2017S
	UPDATE
	00
	

	■ TEMP. CYCLE TEST (Fig.6)
[image: image2.png]2040 minues 2040 ninutes

+65C Thouss \ /

| |

| |

T

0T } } }

1 (.

| [

! L fours
-25C

20~40 ninutes




Fig.6 Illustration of temp. cycle test


	SPECIFICATION
	MODEL NO.
	JHY-1510-50
	P5/5

	ISSUED DATE
	2017-04-08
	
	
	

	REVISION
	2017S
	UPDATE
	00
	

	6. DIMENSIONS (Fig.8)

   Unless otherwise specified, tolerance: ±0.2  (unit: mm)
[image: image3.png]



TABLE OF METERIALS                  



� EMBED  \* MERGEFORMAT ���








PAGE  
- 7 -

_1234567890.dwg

